# Readme file

The following text provides background on the air quality burden analysis for the Illustrative Alternatives.

The EXCEL file in this directory shows the calculations used to determine the pollutant burdens of the Illustrative Alternatives.  The explanation of the calculation process is found in the November 2005 "Evaluation of Illustrative Alternatives on U.S. Side of Border – Volume 2, Technical Analysis" pages 12 and 13.  That text is quoted below.

2.6	Maintain Air Quality

Air pollution burden calculations were done to assess the relative effect of the Illustrative Alternative proposals for afternoon peak hour traffic in the design year of 2035.  “Pollutant burden” means the amount of pollution in terms of mass.  It is not a concentration.  The burden is for international traffic only, i.e., the traffic crossing the border.

Emissions are calculated using a U.S. EPA computer program called MOBILE6.2 (latest version).  The inputs to this model that reflect regional conditions have been checked with SEMCOG.  Emission factors vary by speed and are typically modeled in five mile-per-hour increments (5 mph, 10 mph, 15 mph, etc.).  Most pollutants decrease as speed increases.  Slower speeds generally produce more pollutants per mile, although this relationship becomes more complex at higher speeds.  To generate the pollutant burdens for the Illustrative Alternatives, the average system speed was calculated by dividing the total VMT by the total VHT.

Emission factors were generated for several National Ambient Air Quality Standard pollutants: volatile organic compounds (VOCs), carbon monoxide (CO), nitrogen oxides (NOx), and larger and smaller particulates (PM10 and PM2.5, respectively).  These are pollutants for which the U.S. EPA has set standards to protect public health and welfare.  Emission factors for several air toxics were also calculated: benzene, 1,3-butadiene, formaldehyde, acetaldehyde, and acrolein.  The resulting emission factors for each of these pollutants were multiplied by the difference in vehicle miles as compared to the No Action condition.

Carbon monoxide (CO) concentrations at/near plazas and on crossings and connecting roadway links can be compared to the National Ambient Air Quality Standard (NAAQS) for CO using CAL3QHC, which is a computer program developed by U.S. EPA and the Federal Highway Administration.  It is a dispersion model used to determine CO concentrations at receptor locations downwind of “at-grade,” “fill,” “bridge,” and “cut section” highways located in relatively uncomplicated terrain.  CAL3QHC adds features that account for queuing and, therefore, allows estimates of CO values at specific locations, such as where vehicles pay tolls or pass through customs inspection facilities.  For the Illustrative Alternatives, assumptions were made regarding the plaza, roadway and crossing operations during a typical 2035 afternoon peak traffic hour, related to number of lanes, delay and volumes from the traffic model runs.  The result is that CO in parts per million (ppm) was estimated to be less than two.  The ambient (background) levels of CO in Wayne County in 2005 are between 2.5 and 3.7 ppm.  So, adding the site-specific concentration to the background will not produce CO concentration values greater than the NAAQS standard of 35 ppm.

The balance of the files are MOBILE6.2 input and output files.  Each is in its own subdirectory.